ECE 111 Winter 2022
HW1
Hao Le A15547504

2-to-4 Decoder

Behavioral Level

Dataflow Level

Gate Level

Function of 2-to-4 decoder: Converts a 2-bit binary number into a 4-bit one-hot encoded representation, where the logical 1’s index is representative of the input decimal number.

Number of ALUTs: 4 (6 I/O pins)

A 2-to-4 decoder can be implemented using 4 ALUTs because the 2-bit input has 4 unique states that is one hot encoded into a 4-bit output. Each bit of the output is tied to the output of one ALUT, thus four are needed in total.

Number of Functions: 4 (2 input, 1 output)

Each ALUT takes in the 2-bit input that can take on four unique states that represent 0-3 in decimal. The ALUT’s memory is programmed in a way that will only output a logical 1 if the 2-bit input’s state matches its predesignated output bit index. Since the same 2-bit input is fed into four ALUT’s, and each ALUT outputs a bit, there are six I/O’s.

MUX

Behavioral Level

Dataflow Level

Gate Level

Function of 1-bit MUX: A two-line selector based on a 1-bt binary input. The 1-bit input’s state changes which input line is to be selected and ported to the output i.e. 0 is first line and 1 is second line.

Number of ALUTs: 1 (4 I/O pins)

Since the MUX outputs only one bit, this can be accomplished with one ALUT with three inputs.

Number of Functions: 1 (3 input, 1 output)

2-bit input and 1-bit selector input equates to a 3-bit input Boolean expression. This can be one function that has one output. Overall, only one function is needed.

Full Adder

Behavioral Level

Dataflow Level

Gate Level

Function of full adder: Adds two 1-bit operands and also takes in a 1-bit carry in. The result is two bits, one being the sum, and the other being the carry out. The sum and carry out represent the 2-bit result when the decimal result is larger than 1 that causes a bit overflow in sum.

Number of ALUTs: 2 (5 I/O pins)

Two distinct outputs are required: first between inputs a and b, second between a + b and carry in. This means a total of two ALUTs are needed.

Number of Functions: 2 (2 input, 1 output)

Two functions are needed – these are essentially half adders. Each half adder takes in two 1-bit inputs. The first half-adder’s output is ported to the first input of the second half-adder. This is added to carry in. Overall, there are three inputs and 2 outputs, sum and carry out. A total of 5 I/O’s are present.